HACKATHON CHALLENGE KIT

Synthetic Dataset Report

Predictive & Preventive Failure Intelligence
for Assembly-Line Critical Assets

What the data is, where it 'comes from', and what your solution should find

Data-design assumptions (read me first)

No data was provided with this challenge, so the entire dataset below was designed from scratch to be recognisable to a maintenance practitioner. It is 100% synthetic - every asset, vendor, and value is fictitious. It is procedurally generated with a FIXED random seed (20260909) so the workbook is fully reproducible. The schema in this report matches the workbook (GuardianLine_Synthetic_Dataset.xlsx) exactly, sheet for sheet and field for field.

1  Source-system assumptions

A real plant would not hold this information in one place. The dataset simulates a consolidated export that a maintenance analytics team would assemble by pulling from the systems below and joining them into a single Excel workbook - the most common delivery format for this kind of cross-system, demo-scale analysis.

| Simulated source system | Sheet | What it represents |
| --- | --- | --- |
| Asset register (EAM / CMMS) | Asset_Master | The plant's asset management system - the master list of critical equipment, criticality class, and service policy. |
| Machine / line controllers (MES) | Usage_Log | Manufacturing Execution System - cycle counts, start-stop events, and duty cycle per asset. |
| Condition-monitoring historian (SCADA / IoT) | Sensor_Readings | Time-series historian - temperature, vibration, current, pressure, and cycle-time, aggregated weekly. |
| Work-order system (CMMS) | Maintenance_History | Preventive, corrective, and inspection work orders with recorded failure modes. |
| Production planning (ERP / MES) | Production_Context | Line and shift demand context - planned vs. actual volume, overtime, model mix. |
| Quality management system (QMS) | Quality_Linkage | Per-asset quality outcomes - defects, scrap, and rework, for linking asset health to quality. |

Why Excel, and why weekly

Excel is the realistic lowest-common-denominator handoff for a cross-system, demo-scale extract - it needs no database and every team can open it. Signals are aggregated to WEEKLY snapshots (three weeks: 2026-08-03, 08-10, 08-17) to keep the dataset demo-sized while still showing trends that develop over time.

2  Entity model, keys & relationships

Six sheets link through a small set of keys. The most valuable insights only appear when sheets are joined - some seeded problems are invisible in any single sheet.

Primary keys and join keys

| Sheet | Primary key | Foreign / join key | Grain |
| --- | --- | --- | --- |
| Asset_Master | AssetID | - | The hub. One row per Class A asset. |
| Usage_Log | UsageID | AssetID + WeekStarting | Weekly usage per asset. |
| Sensor_Readings | ReadingID | AssetID + WeekStarting | Weekly signals per asset. |
| Maintenance_History | WorkOrderID | AssetID | Work orders per asset. |
| Production_Context | ContextID | Line + WeekStarting | Joins to assets via Asset_Master.Line. |
| Quality_Linkage | QualityID | AssetID + WeekStarting | Weekly quality per asset. |

Join map at a glance

Asset_Master (AssetID) is the centre. Usage_Log, Sensor_Readings, Maintenance_History, and Quality_Linkage all attach by AssetID. Production_Context attaches by Line + WeekStarting through Asset_Master.Line. Watch for referential-integrity breaks: at least one work order references an AssetID that does not exist in Asset_Master.

3  Field-by-field schema

Every field in every sheet is documented below. Row counts reflect the shipped workbook.

3.1  Asset_Master  (45 rows)

| Field | Type | Description |
| --- | --- | --- |
| AssetID | Text (PK) | Unique asset identifier, e.g., A001. Primary key. |
| AssetName | Text | Human-readable asset name. |
| AssetType | Text | Equipment type (weld robot, servo press, conveyor drive, etc.). |
| Class | Text | Criticality class. All rows are 'A' (business-critical). |
| Line | Text | Assembly line the asset belongs to (BODY-1, PAINT-1, ASSY-2, ...). |
| Station | Text | Station location on the line. |
| Manufacturer | Text | Equipment maker (fictitious use of common brand names). |
| InstallDate | Date (ISO) | Commissioning date. |
| RatedCycleLife | Integer | Manufacturer-rated cycle life for the asset type. |
| CumulativeCycles | Integer | Cycles accumulated to date. Compare against RatedCycleLife for wear. |
| BusinessImpactScore | Integer 6-10 | How disruptive a failure is. Drives prioritisation. |
| ReplacementCost_EUR | Integer | Approximate replacement cost in EUR. |
| ReplacementLeadDays | Integer | Lead time to source a replacement (days). |
| MaintIntervalDays | Integer | Planned preventive-maintenance interval (days). |

3.2  Usage_Log  (135 rows)

| Field | Type | Description |
| --- | --- | --- |
| UsageID | Integer (PK) | Unique usage record id. |
| AssetID | Text (FK) | Links to Asset_Master.AssetID. |
| WeekStarting | Date (ISO) | Monday of the reporting week. |
| CyclesRun | Integer | Cycles run during the week. |
| StartStopEvents | Integer | Number of start/stop events - a key stress driver. |
| PeakThroughput_UPH | Integer | Peak throughput (units per hour) during the week. |
| DutyCyclePct | Integer | Percent of available time the asset was actively running. |
| CumulativeCyclesEOW | Integer | Running cumulative cycles at end of week. |

3.3  Sensor_Readings  (133 rows)

| Field | Type | Description |
| --- | --- | --- |
| ReadingID | Integer (PK) | Unique reading record id. |
| AssetID | Text (FK) | Links to Asset_Master.AssetID. |
| WeekStarting | Date (ISO) | Reporting week. NOTE: some assets are missing a week (data gap). |
| AvgTempC | Decimal | Average operating temperature (degC). |
| MaxTempC | Decimal | Peak temperature (degC). |
| AvgVibration_mm_s | Decimal | Average vibration (mm/s). |
| MaxVibration_mm_s | Decimal | Peak vibration (mm/s). |
| AvgCurrent_A | Decimal | Average motor current draw (A). |
| MaxCurrent_A | Decimal | Peak current draw (A). |
| AvgPressure_bar | Decimal | Average hydraulic/pneumatic pressure (bar). |
| CycleTime_s | Decimal | Average cycle time (s). Rising cycle time can signal degradation. |
| ScenarioFlag | Text | Non-blank on seeded rows (hint only). Do not rely on this at judging. |

3.4  Maintenance_History  (103 rows)

| Field | Type | Description |
| --- | --- | --- |
| WorkOrderID | Text (PK) | Work-order id. NOTE: one value is intentionally duplicated. |
| AssetID | Text (FK) | Links to Asset_Master.AssetID. NOTE: one value is an orphan (no master). |
| Date | Date (ISO) | Work-order date. |
| MaintType | Text | Preventive, Corrective, or Inspection. |
| FailureMode | Text | Recorded failure mode (blank for non-corrective work). |
| DowntimeHours | Decimal | Downtime caused by the work order (hours). |
| PartsReplaced | Text | Part replaced, or 'None'. |
| TechNotes | Text | Free-text technician note. |

3.5  Production_Context  (45 rows)

| Field | Type | Description |
| --- | --- | --- |
| ContextID | Text (PK) | Unique context record id. |
| Line | Text | Assembly line (joins to Asset_Master.Line). |
| WeekStarting | Date (ISO) | Reporting week. |
| Shift | Text | Shift (S1/S2/S3). |
| ModelMix | Text | Dominant model mix that week (sedan-heavy, SUV-heavy, EV-pilot, balanced). |
| PlannedVolume | Integer | Planned units for the line/shift/week. |
| ActualVolume | Integer | Actual units produced. |
| OvertimeHours | Integer | Overtime hours worked - a proxy for demand stress. |
| DemandIndex | Decimal | Relative demand pressure (1.0 = nominal). |

3.6  Quality_Linkage  (135 rows)

| Field | Type | Description |
| --- | --- | --- |
| QualityID | Text (PK) | Unique quality record id. |
| AssetID | Text (FK) | Links to Asset_Master.AssetID. |
| WeekStarting | Date (ISO) | Reporting week. |
| DefectCount | Integer | Defects attributed to the asset that week. |
| DefectType | Text | Dominant defect type (weld porosity, misalignment, torque, ...). |
| ScrapRate_pct | Decimal | Scrap rate (%). |
| ReworkCount | Integer | Units requiring rework. |
| LinkedFailureMode | Text | Populated where a quality spike coincides with asset degradation. |

4  Anomaly / scenario catalog

These are the targets your solution should detect or act on. They span single-signal trends, usage-driven stress, maintenance gaps, cross-sheet correlations, and data-quality faults. Several are only visible when sheets are JOINED - those reward teams that model the data properly. Seeded rows are shaded amber in the workbook as a hint, but the catalogue below - not the shading - is the definition.

| Code | Scenario | What your solution should detect / do |
| --- | --- | --- |
| S1 | Overheating trend | Progressive temperature rise -> overheating failure-mode hypothesis |
| S2 | Abnormal vibration | Vibration drift -> bearing / mechanical wear hypothesis |
| S3 | Excess current draw | Current climb -> motor load / friction hypothesis |
| S4 | Cycle-time drift | Slowing cycle time -> degradation / calibration drift |
| S5 | Pressure drop | Falling pressure -> seal leak / hydraulic loss |
| S6 | Accelerated wear | Cumulative cycles near rated life -> end-of-life risk |
| S7 | Restart / stop-start stress | High start-stop under high duty -> stress-driven risk |
| S8 | Maintenance overdue | Days since last preventive >> interval -> overdue |
| S9 | Quality correlation | Defect spike coincident with asset degradation (join reward) |
| S10 | Orphan maintenance record | Referential-integrity break, only visible on join |
| S11 | Sensor data gap | Missing week -> data-health / completeness flag |
| S12 | Duplicate work order | Duplicate primary key -> data-quality flag |
| S13 | High-impact / moderate-risk conflict | Prioritization must weigh business impact, not raw risk alone |

Cross-system cases (only visible on join)

S10 Orphan maintenance record - a work order references an AssetID that is not in Asset_Master. A single-sheet view will never see it; a join will.

S11 Sensor data gap - two assets are missing their middle-week sensor row. A completeness check across the expected weeks reveals the gap.

S9 Quality correlation - a defect/scrap spike in Quality_Linkage lines up with a degrading signal in Sensor_Readings for the same asset and week.

S13 Impact vs. risk - a moderate-risk asset with a very high BusinessImpactScore should out-prioritise a higher-risk but low-impact asset.

For facilitators: the answer key

The workbook contains a hidden sheet named ANSWER_KEY listing every seeded scenario, its code, the affected assets/location, the signal or sheet involved, and what teams should detect. Unhide it via right-click on any sheet tab > Unhide. Do NOT share it with participants. Because teams may be scored on a modified copy, solutions must generalise rather than hard-code these IDs.

5  Using the workbook

| Item | Note |
| --- | --- |
| README sheet | The first sheet explains purpose, sheets, join keys, hint shading, and the data notice. Start there. |
| Hint shading | Amber-shaded rows/cells mark seeded scenarios. This is a learning aid, not a specification - build for the general case. |
| Fixed seed | The data is generated with seed 20260909; regenerating produces identical values. |
| No formulas | All cells are literal values - there are zero formulas and therefore zero formula errors. |
| Reproduce / extend | Facilitators can change the seed or add scenarios to produce a fresh copy for scoring; the schema stays identical. |

Schema fidelity

This report was generated directly from the shipped workbook, so the sheet names, field names, and row counts above match the .xlsx exactly. If you extend the data, keep the field names stable so downstream joins continue to work.